DEGREE PROJECT PROPOSAL

INDUSTRIAL ENGINEERING

SEMESTER 2026-20

Santiago Reyes Cortes

202221394

Advisor: Alejandra Tabares Pozos

1. PROJECT TITLE (15-20 words)

Optimization of the operation of an electric vehicle charging station in Colombia through simulation and smart charging algorithms, with infrastructure guidelines for future demand-growth scenarios.

2. PROBLEM STATEMENT / NEED

2.1. Current Situation (Identification/Characterization Stage)

The first electric cars appeared in the 19th century, and in 2008 Tesla Motors launched the Tesla Roadster, demonstrating that electric cars could achieve high performance and a range of more than 320 km. Throughout the 2010s, continuous improvements in lithium-ion battery technology helped reduce costs and improve range, making this type of vehicle much more affordable for the mass market.

From 2020 to the present, the combination of strict environmental regulations, government incentives, and the need to reduce greenhouse gas emissions has driven a massive boom in both supply and demand.

This year, Colombia's electric vehicle market is experiencing a historic boom, growing 171.3% in the first quarter compared to 2025, driven by tax benefits, exemption from driving restrictions ("pico y placa"), and a wide range of brands such as BYD, Toyota, Kia, Volvo, and Tesla. Buyers are seeking savings on fuel and maintenance, in addition to driving-restriction exemptions and vehicle tax discounts.

Despite this growth, the infrastructure installed in Colombia — 447 public stations and 844 connectors registered by the Ministry of Mines and Energy as of early 2026 — often operates below its potential. Existing stations show long wait times during peak hours, suboptimal power allocation among chargers, poor coordination with energy prices, and, under high simultaneous-demand scenarios, a risk of exceeding the local transformer's operating limits. For this reason, before scaling up the construction of new infrastructure, it is critical to understand and improve the operation of already-installed stations.

2.2. Design (Text or Table)

This problem calls for an engineering design, understood as an iterative, creative, decision-making process aimed at improving the operation of an existing charging station in Colombia. The design will address power allocation among chargers, charging-time scheduling, user-queue management, and coordination with price signals and the local transformer's limits, seeking to raise user satisfaction and operator profitability without compromising the distribution network. In addition, the design will leave room to propose infrastructure improvement guidelines (capacity, number of chargers, connectors, integration with distributed generation) to address future demand-growth scenarios.

The design of the operating strategy — and of the future infrastructure guidelines — must take into account operating-cost constraints, protocol interoperability (OCPP, CCS Combo 2), available contracted power, legal and electrical-safety considerations, ease of maintenance, and applicable national policies and regulations (Law 1964 of 2019, RETIE, CREG resolutions).

2.3. National and International Regulations and Standards (Text or Table)

| # | Regulation / Instrument | Issuing Entity | Description |
| --- | --- | --- | --- |
| 1 | Law 1964 of 2019 | Congress of the Republic | Framework law on electric mobility. Defines fast (>50 kW) and slow (7-49 kW) charging stations. Requires special-category municipalities to guarantee a minimum of 5 public stations (Bogota: min. 20). Sets a maximum vehicle-tax rate of 1%, exemption from driving restrictions ("pico y placa"), and a 10% discount on mandatory vehicle insurance (SOAT). |
| 2 | RETIE - Resolution 40117 of 2024 | Ministry of Mines and Energy | Mandatory Electrical Installations Technical Regulation. Classifies EV chargers as a product subject to conformity assessment. Requires grounding, electrical protection, signage, and complete project documentation. |
| 3 | Resolution 40223 of 2021 | Ministry of Mines and Energy | Establishes the minimum connector standard for public stations: at least one Type 1 connector (SAE J1772) for Level 2 and 3 AC charging, and one CCS Combo 1 connector for fast DC charging. Requires registration on the Ministry's platform and compliance with RETIE. |
| 4 | Resolution 40123 of 2024 | Ministry of Mines and Energy | Requires all publicly accessible stations to connect via an open protocol (OCPP or an equivalent ISO/IEC/CONTEC standard) and to report operational data (location, connector type, status) to the Ministry's platform. |

Table 1. National Regulations

| # | Regulation / Instrument | Issuing Entity | Description |
| --- | --- | --- | --- |
| 1 | IEC 61851 | International Electrotechnical Commission (IEC) | Safety and control standard for conductive charging. Defines the four charging modes, mechanical, electrical, and electromagnetic-compatibility (EMC) requirements, and the signaling protocols (PWM control pilot) between the vehicle and the station. |
| 2 | IEC 62196 | International Electrotechnical Commission (IEC) | Specifies the physical design of connectors, sockets, and vehicle inlets (Type 1, Type 2, CCS). Colombian regulation requires a Type 2 connector for AC charging and a CCS Combo 2 connector for fast DC charging, both compliant with this standard. |
| 3 | ISO 15118 | International Organization for Standardization (ISO) | Defines digital communication between the vehicle and the grid. Enables Plug & Charge (automatic authentication upon connection), smart energy management, and Vehicle-to-Grid (V2G) communication for integration with the smart electrical grid. |
| 4 | OCPP v2.0.1 | Open Charge Alliance (OCA) | Open protocol for remote monitoring, firmware updates, and centralized management of stations from different manufacturers. Includes communication encryption and native compatibility with ISO 15118. Required by Colombian regulation (Res. 40123/2024). |

Table 2. International Regulations

2.4. Background (Identification/Characterization Stage)

Three articles related to the topic were consulted. The first background reference presented is “EV2Gym Simulator for Smart V2G Charging.”

This article proposes EV2Gym, an open-source simulation platform aimed at developing and evaluating smart charging algorithms in Vehicle-to-Grid (V2G) scenarios. The central problem it addresses is the lack of a standardized environment for comparing different types of solutions — heuristics, mathematical programming, and reinforcement learning (RL) — under realistic and reproducible conditions.

The authors built the simulator using real data: user-behavior profiles come from the ElaadNL platform (Netherlands), vehicle specifications were obtained from the RVO-NL report with records of more than 200,000 vehicles, and energy prices correspond to historical ENTSO-E spot-market data. The simulator models battery dynamics in detail through a two-stage model (constant current and constant voltage), incorporates a battery degradation model with calendar- and cycle-aging components, and adds transformer constraints, inflexible loads, and photovoltaic generation.

Regarding quantitative results, the authors evaluated different algorithms under two reference scenarios. In the power setpoint-tracking (PST) problem with 20 chargers, the Round Robin heuristic achieved an energy error of only 57 kWh compared to 313 kWh for the unmanaged algorithm (AFAP), with 99% user satisfaction. In the V2G profit-maximization problem with 10 bidirectional chargers, Model Predictive Control (MPC) achieved the best balance between profit (€17.8) and user satisfaction (76%), while the RL algorithm TD3 achieved the highest gross profit (€31.1) but with the lowest user satisfaction (38%). The authors highlighted that RL methods run up to 1,000 times faster than MPC (0.009 s versus 108 s per step), making them more attractive for large-scale real-time operation.

This work is directly relevant to the project because it establishes a methodological and computational framework on which charging-infrastructure expansion and management strategies can be built. In particular, the user arrival/departure behavior models, the scenario classification (residential, workplace, and public), and the proposed evaluation metrics are transferable to the Colombian context with the necessary adaptations.

The second background reference presented is “Physics-Informed Reinforcement Learning for Large-Scale EV Charging.”

This work addresses a critical limitation of previous approaches: most RL algorithms for EV charging ignore the physical constraints of the electrical distribution network, which causes voltage violations when deployed at real scale. The proposed solution is the PI-TD3 algorithm (Physics-Informed Twin Delayed Deep Deterministic Policy Gradient), which incorporates the network's power-flow equations directly into the RL agent's training process through differentiable reward signals.

The methodology combines three elements: a differentiable power-flow model based on the network's admittance matrix, each vehicle's battery dynamics as a known, differentiable function, and a forward simulation horizon of K steps that allows gradients to be propagated through the system's physics. Experiments were carried out on the IEEE 34-bus and 123-bus test networks, simulated using the EV2Gym environment and the RL-ADN power-flow module.

The results are especially significant in terms of scale and quality. On the 34-bus network with 150 charging points, PI-TD3 reduced per-bus voltage violations by 20% relative to classic TD3 (from 127.8 to 104.2 violations) and achieved 95.6% user satisfaction, only 4 percentage points below the oracle MPC (which assumes perfect knowledge of the future). It also converged roughly four times faster than conventional TD3 during training. On the 123-bus network with 500 simultaneous electric vehicles, PI-TD3 matched or outperformed MPC in total voltage violations and cumulative reward, with an execution time of only 10 ms per step versus several minutes for MPC. The algorithm also demonstrated robustness under conditions not seen during training: evaluated on scenarios with loads between 0.5x and 1.25x nominal demand, it maintained user satisfaction above 90% without any retraining.

For the present project, this background reference has a twofold importance. On one hand, it shows that the massive expansion of charging points in a city — such as the one proposed for Colombia — requires not only the physical installation of infrastructure but also smart management strategies that respect the electrical network's operating limits. On the other hand, the result that a 123-bus network can manage 500 vehicles in real time with an algorithm executable in 10 ms opens the door to viable technological solutions for grid operators and charging-service providers in Colombia.

Finally, the reference “Microgrids as a Basis for the Integration of Distributed Energy Resources” is presented.

This chapter reviews the concept of a microgrid as a distributed generation network capable of operating either connected to the main grid or in island mode, and analyzes its three-level hierarchical control: the primary level stabilizes voltage and frequency in real time, the secondary level corrects deviations, and the tertiary level optimizes power flows. Distributed resources — photovoltaic panels, batteries, microturbines — are coordinated through a central controller (MGCC) under centralized, decentralized, or agent-based architectures.

Its relevance to this project lies in the fact that each high-power charging station acts as a dynamic demand node within the local distribution network. Understanding this control architecture is essential for identifying strategic locations that do not overload existing transformers or generate voltage instabilities — constraints that must be considered from the solution's design stage onward.

2.5. Desired Situation

Through this project, an optimized operating strategy is expected to be designed and evaluated for an already-existing charging station, using an academic scenario in an open GitHub environment, that simultaneously improves user satisfaction, operator revenue, and compliance with the local network's technical limits. Using the EV2Gym tool, realistic operating scenarios will be simulated — with arrival profiles, demand patterns, and energy prices — to compare charging-management algorithms (heuristic, MPC, and reinforcement learning) against the current unmanaged operation.

As a concrete deliverable, the project is expected to produce a simulation-validated operating strategy for a representative charging station in Colombia, together with a proposal for infrastructure guidelines that would allow the station's capacity to be scaled up (number of chargers, installed power, possible distributed resources) to address future demand-growth scenarios. These guidelines could serve as a replicable reference for other stations in special-category cities (Bogota, Medellin, Cali, Barranquilla, and Bucaramanga) under the framework of Law 1964 of 2019.

Through simulations on the EV2Gym platform, the designed solution is expected to reach a user satisfaction above 90% — a threshold comparable to that obtained by smart-management algorithms in reference international studies (Orfanoudakis et al., 2025) — and a reduction of the unserved-energy error to below 15% relative to the current unmanaged operating scenario of the analyzed station. Additionally, the optimized operation is expected not to generate voltage violations in the local distribution network, remaining within the ±5% nominal-voltage regulatory limit established by RETIE. The infrastructure guidelines for future scenarios will be evaluated against the same metrics under projected demand increases.

If the project is executed as planned, the pilot station will perform better than its current baseline, and its results will be replicable across the country's registered public stations. Likewise, the proposed infrastructure guidelines will help guide an orderly capacity expansion in the face of an estimated 60% demand growth in major cities, thereby contributing to the National Development Plan's goal of consolidating the country's just energy transition.

2.6. Analysis of Consequences of the Final Situation

Implementing an expanded network of EV charging points in Colombia would generate simultaneous impacts across multiple dimensions. Environmentally, the main benefit would be reduced CO2 emissions in urban areas, contributing to the Paris Agreement's goals; however, the increase in electricity demand could increase pressure on non-renewable generation sources during periods of low hydro availability. Economically, EV owners and private station operators would be the main beneficiaries, although the initial infrastructure investment represents a barrier for municipalities with lower fiscal capacity. Politically, the project aligns with the 2022-2026 National Development Plan and strengthens Colombia's position as a regional benchmark in the energy transition, although its sustainability depends on the continuity of current tax incentives, which could change with a change in government. Socially, the expansion would initially benefit middle- and upper-income EV owners, which could deepen access gaps if not accompanied by inclusion policies. Technologically, the adoption of open standards such as OCPP and CCS Combo 2 would favor interoperability across the charging ecosystem, although already-installed infrastructure with incompatible connectors would generate conversion costs for some operators.

[Template instruction from the university form, not part of the proposal content:] Complete the "Research Ethics" course modules on Bloque Neon and submit the completion certificate (see instructions).

COURSES / TOOLS / METHODOLOGY TO BE USED IN THE PROJECT

| Area | Course | Code | Used | Description of the course tools to be integrated into your project |
| --- | --- | --- | --- | --- |
| Operations Research and Statistics | OPTIMIZATION PRINCIPLES | IIND-2103 | Yes | Modeling techniques, solution techniques, and algorithmic design using modern mathematical programming packages. |
| Operations Research and Statistics | PROBABILITY AND STATISTICS I | IIND-2106 | Yes | Statistical foundations for correlation analysis and model validation. |
| Operations Research and Statistics | PROBABILITY AND STATISTICS II | IIND-2107 | Yes | Statistical foundations for correlation analysis and model validation, integrating analytics and automated decision-making. |
| Operations Research and Statistics | DISCRETE EVENT SIMULATION | IIND-3113 | Yes | Evaluation of alternative performance scenarios through simulation. |
| Production and Logistics | PRODUCTION CONTROL | IIND-2201 | No | Not applicable |
| Production and Logistics | PRODUCTION FUNDAMENTALS | IIND-2202 | No | Not applicable |
| Production and Logistics | LOGISTICS | IIND-3221 | Yes | Use of key strategic elements affecting supply-chain performance (EV charging network), including: customer service, network and facility design, supply, distribution, and information technologies. |
| Economics and Finance | MANAGEMENT CONTROL SYSTEMS | IIND-2400 | No | Not applicable |
| Economics and Finance | INVESTMENT DECISION ANALYSIS | IIND-2401 | Yes | Competencies needed to identify, size, and incorporate key variables in structuring and financially evaluating investment projects. Evaluate the project's implications in a global, economic, and social context. |

Table 3. List of Courses to Be Used in the Degree Project

2.7. Research Question (Formulation Stage)

How can the operation of an existing electric vehicle charging station in Colombia be optimized — in terms of user satisfaction, operator profitability, and compliance with local network limits — and what infrastructure guidelines are needed to scale that operation under future scenarios of demand growth?

3. PROJECT JUSTIFICATION (2 paragraphs)

This project arises from the accelerated growth of Colombia's electric vehicle market, driven by government incentives, tax benefits, and greater environmental awareness. However, this increase in demand has not been matched by an efficient operation of the existing charging infrastructure. Although the country has hundreds of installed stations, these show problems such as long wait times, inefficient power allocation, and risks of overloading the local electrical network. In this context, the problem lies not only in the lack of infrastructure, but in the need to optimize the use of already-available resources. This topic was therefore chosen with the aim of addressing a critical bottleneck in the country's energy transition, especially in large cities where simultaneous charging demand keeps growing.

Research on this topic is fundamental, as it allows more efficient operating strategies to be analyzed and evaluated through simulation tools and smart charging algorithms. Unlike traditional approaches focused on infrastructure expansion, this project proposes optimizing the operation of existing stations using advanced models that consider both user demand and the electrical network's technical constraints. The added value lies in integrating methodologies such as simulation in realistic environments with heuristic, model predictive control, and reinforcement learning algorithms, which makes it possible to generate replicable and scalable solutions for the Colombian context. In this way, the project not only contributes to improving the user experience and operator profitability, but also provides strategic guidelines for a sustainable and efficient expansion of the country's charging infrastructure.

4. OBJECTIVES

4.1. General Objective

To design and evaluate, through simulation on the EV2Gym platform, an optimized operating strategy for an electric vehicle charging station that improves user satisfaction and operator profitability while respecting the local network's technical limits, and to formulate infrastructure guidelines that allow this operation to be scaled under future demand scenarios.

4.2. Specific Objectives

• Characterize the current operation of a representative charging station in Colombia, identifying baseline metrics (waiting times, unserved energy, revenue, and demand profiles).

• Model the station and its environment on the EV2Gym platform, incorporating local transformer constraints and energy prices, and validate the model against real operation.

• Implement and compare at least three charging-management algorithms (heuristic, MPC, and reinforcement learning) against unmanaged operation, using user satisfaction, revenue, and voltage violations as criteria.

• Propose a recommended operating strategy and formulate infrastructure guidelines (capacity, number of chargers, interoperability, and possible integration with distributed generation) for future scenarios of increased demand.

• Evaluate the replicability of the proposed strategy in stations located in special-category cities across the country, identifying critical constraints and assumptions.

5. ACTIVITY SCHEDULE

| Activity | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Literature review | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Problem definition | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. User research |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 4. Current-process analysis |  |  |  | X | X | X |  |  |  |  |  |  |  |  |  |  |
| 5. Requirements definition |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |
| 6. Conceptual design |  |  |  |  |  | X | X | X | X |  |  |  |  |  |  |  |
| 7. Prototype development |  |  |  |  |  |  | X | X | X | X | X | X | X | X |  |  |
| 8. Prototype testing |  |  |  |  |  |  |  |  | X | X | X | X | X | X |  |  |
| 9. User validation |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |
| 10. Project documentation | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |

Table 4. Activity Schedule (weeks 1-16). Reproduced from the original Gantt chart; since the source table was an embedded image in the PDF, please double-check the exact week alignment against your own file before final submission.

6. REFERENCE LIST (Minimum 3)

Note: reference titles are kept in their original publication language, since translating a citation's title would misrepresent the source.

• Autosdeprimera. (2025, noviembre 7). La venta de vehículos eléctricos en Colombia crece un 93,7% en octubre. https://autosdeprimera.com/cifras-del-mercado/nacional/la-venta-de-vehiculos-electricos-en-colombia-crece-un-937-en-octubre/

• Enel X. (2022, abril 21). ¿Cuándo se creó el primer carro eléctrico? Descubre su historia. https://www.enelx.com/co/es/historias/historia-del-carro-electrico

• Fundación Endesa. (s. f.). eMobility: coches eléctricos. https://www.fundacionendesa.org/es/educacion/endesa-educa/recursos/emobility-coches-electricos

• International Energy Agency (IEA). (2024). Global EV Outlook 2024: Trends in electric cars. https://www.iea.org/reports/global-ev-outlook-2024/trends-in-electric-cars

• Orfanoudakis, S., Diaz-Londono, C., Yılmaz, Y. E., Palensky, P., & Vergara, P. P. (2025). EV2Gym: A flexible V2G simulator for EV smart charging research and benchmarking. IEEE Transactions on Intelligent Transportation Systems, 26(2), 2410-2421.

• Orfanoudakis, S., Oliehoek, F. A., Palensky, P., & Vergara, P. P. (2025). Physics-informed reinforcement learning for large-scale EV smart charging considering distribution network voltage constraints. arXiv preprint arXiv:2510.12335v2.

• Mahmoud, M. S. (2017). Microgrid control problems and related issues. In M. S. Mahmoud (Ed.), Microgrid (pp. 1-42). Elsevier. https://doi.org/10.1016/B978-0-08-101753-1.00001-2