Final Capacity Brief — Parameter, Method, and Implementation Justification: DC Session Duration and Growth Scenario

**Status: complete (2026-10-07). Run unattended on branch semana-7;

nothing committed.**

This document follows the Week 5–7 and closure mechanism. It is a Markdown

record rendered to .docx by scripts/render_docx.py. The decision log,

with the Bogotá time of each checkpoint, is the dwell section at the top of

thesis_docs/overnight_report.md. The baseline was HEAD 942122a with a

clean working tree (thesis_docs/dwell_baseline_status.txt).

Headline Results Summary

**1. EV2Gym's session durations are not DC sessions (the Checkpoint A rule

fired).**

The simulated mean connection is 300.6 min [296.5, 304.7]. No session

is shorter than 225 min.

The reference is 42 min for paid DC charging (U.S. DOE, 2023), and the

rule's ceiling is 90 min.

The 42-minute DC model therefore became primary. The closure results are

a declared sensitivity.

**2. The closure's 34.6% demand not served was largely a session-duration

artefact.** At the same spawn multiplier, DC sessions give:

rejected arrivals of 0.9/day instead of 8.9;

port occupancy of 14% instead of 35%;

demand not served of 2.4% (AFAP) and 5.0% (transformer-aware Round Robin).

**3. EV2Gym's Round Robin follows a median-smoothed power setpoint, not the

transformer.**

Its zero overload in Weeks 1–7 came from long stays.

Under 42-minute sessions it fails every criterion at every demand tested:

at 1.0×, DNS is 24.8% and 65/100 runs overload.

AFAP and the final RL model (out of its training distribution) also

exceed 100 kW from the lowest level.

The binding constraint is power, not ports.

4. Threshold with a transformer-aware round-robin load manager

(added as a diagnostic in this brief; promoted to the recommended strategy, 2026-10-07):

**8 ports and the 100 kW (112.5 kVA) unit serve up to 47 arrivals/day

(763 kWh/day) with DNS ≤ 15%** (9.0%, CI high 10.2%; n_clusters = 50).

The threshold is the same at 32 min and falls to 35 arrivals/day at

78 min.

5. Growth scenario (42 min).

| Level | Round Robin (EV2Gym) | Transformer-aware Round Robin | AFAP |
| --- | --- | --- | --- |
| 1.3× (47 arrivals/day) | none | 8 ports, 100 kW | 8 ports, 300 kVA |
| 1.6× (57 arrivals/day) | 14 ports, 400 kVA | 10 ports, 100 kW | 10 ports, 300 kVA |

6. Final compliance (DC 42 min, 8 ports, 100 kW).

Only the transformer-aware Round Robin meets every evaluable target at 1.0× and

1.3×.

At 1.6× no arm meets the satisfaction target counting rejected arrivals.

Voltage is not evaluable under DC sessions.

Part 1 — Parameters and Methods

| Parameter | Value | Label | Justification | Citation |
| --- | --- | --- | --- | --- |
| Central DC session duration | 42 min (lognormal mean) | external reference, not Colombian | Enel's service is paid (pay per kWh), so paid sessions are the analogue. It is a measured mean over 1,412,050 sessions. It lies between Enel's 50% (25–30 min) and full-charge (just over 1 h) times. | U.S. DOE (2023); Blu Radio (2026) |
| Low bracket | 32 min | external reference, not Colombian | Self-reported recall of the last session (survey, 3,350 households), so it is the low bound, not the centre | Hardman (2026) |
| High bracket | 78 min | external reference, not Colombian | Mean of free US DC sessions (957,265). Without a price there is no incentive to leave, so it bounds the result from above | U.S. DOE (2023) |
| Coefficient of variation | 0.5 | labelled assumption | Survey per-activity SDs: 15.56 / 15.06 / 19.00 min on means of 29.65 / 30.70 / 38.39, a CV of 0.49–0.52 | Hardman (2026), Suppl. Table 3 |
| Checkpoint A rule | 90 min | brief's rule; plausibility ceiling | The slowest Enel figure (about 1 h 30 min to 100%, earlier equipment). It describes a full charge, not a typical session. The page could not be saved | Enel Colombia (2024) |
| Rounding | nearest 15-min step, floor 1 step | simulator constraint | ±7.5 min resolution; 13% of 42-min sessions are one step | dc_sessions.py |
| Energy per session | not rescaled (16.1 kWh simulated) | declared limitation | 27% below the 22.0 kWh paid-DC reference. It comes from the Dutch per-arrival demand table, not from the battery | U.S. DOE (2023) |
| Breaking criteria | satisfaction counting rejected arrivals CI low ≥ 90%; DNS CI high ≤ 15%; P95 peak CI high ≤ rating (kW) | brief; hardest CI side (conservative) | The same as the closure run; the peak is judged against the candidate's own rating in C2 | analyze_dwell_capacity.py |
| Transformer ratings | 100 kW reference; 150 / 225 / 300 / 400 kVA at pf 0.894 | standard classes | Enel ET-013 Table 1; pf from CREG 015/2018 (closure run) | SOURCES_closure.md |
| Port variants | 8–16 at constant station demand (spawn × 8/P) | closure convention | EV2Gym draws arrivals per port | closure run |
| Round Robin, transformer-aware (added as a diagnostic; recommended from 2026-10-07) | Round Robin, budget 0.999 × rating | labelled judgement call | Separates round-robin load management from EV2Gym's setpoint coupling | ev2gym_thesis/heuristics.py |

The 42-minute justification (as written in chapter 06, S6.6.2).

1. Payment model. Enel's public charging in Bogotá is paid: users

activate the charge and pay only for the energy consumed (Blu Radio,

2026). Paid DC sessions are therefore the behavioural analogue. Free

sessions last almost twice as long, 78 against 42 minutes (U.S. DOE,

2023), because without a price users have no incentive to leave.

2. Strength of the evidence. The 42-minute figure is the mean of

1,412,050 measured paid DC sessions (U.S. DOE, 2023). The 32-minute

California figure is self-reported recall (Hardman, 2026), so it is the

low bound.

3. Consistency with Enel's own figures. Unicentro reaches about 50% in

25–30 minutes and a full charge in a little over an hour (Blu Radio,

2026). A 42-minute mean lies between the two, consistent with users

stopping before 100%. The older 1 h 30 min figure (Enel Colombia, 2024)

is a full charge on earlier equipment, so it is used only as the rule's

ceiling.

4. Sensitivity bracket. The transformer-aware Round Robin's threshold is the same

at 32 and 42 min (holds at 1.3×) and moves down one level at 78 min

(holds at 1.0×): about 49 / 47 / 35 arrivals per day. EV2Gym's Round

Robin meets the criteria at no level under any duration.

5. Declared limitation. No Colombian per-session statistics are

published. The value comes from the US fleet and charger mix. It

replaces Dutch AC times that are inconsistent with DC charging, but it

is not Colombian data (chapter 08).

Final target compliance (DC 42 min, 8 ports, 100 kW;

results/dwell_e_target_compliance.csv; n_clusters = 50). Satisfaction and

DNS are judged on the hardest CI side.

| Level | Arm | Satisfaction, served | Satisfaction, counting rejected | ENS_rel, served | DNS < 15% | Transformer | Voltage ±5% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0× | AFAP | met | met (97.9%) | met (0.0%) | met (2.4%) | not met (97/100) | not evaluable |
| 1.0× | Round Robin (EV2Gym) | met | met (93.0%) | not met (22.9%) | not met (24.8%) | not met (65/100) | not evaluable |
| 1.0× | Final RL (out of distribution) | met | met (92.2%) | not met (26.3%) | not met (28.1%) | not met (81/100) | not evaluable |
| 1.0× | Round Robin, transformer-aware | met | met (97.3%) | met (2.7%) | met (5.0%) | met (0/100) | not evaluable |
| 1.3× | AFAP | met | met (94.7%) | met | met (5.5%) | not met (100/100) | not evaluable |
| 1.3× | Round Robin (EV2Gym) | met | not met (90.6%, CI low 89.6%) | not met (19.0%) | not met (23.6%) | not met (83/100) | not evaluable |
| 1.3× | Final RL | met | not met (89.4%) | not met (24.8%) | not met (29.0%) | not met (95/100) | not evaluable |
| 1.3× | Round Robin, transformer-aware | met | met (93.9%) | met (3.7%) | met (9.0%) | met (0/100) | not evaluable |
| 1.6× | AFAP | met | not met (89.5%) | met | met (10.5%) | not met (100/100) | not evaluable |
| 1.6× | Round Robin (EV2Gym) | met | not met (86.1%) | not met (16.6%) | not met (25.3%) | not met (96/100) | not evaluable |
| 1.6× | Final RL | met | not met (84.9%) | not met (22.5%) | not met (30.6%) | not met (98/100) | not evaluable |
| 1.6× | Round Robin, transformer-aware | met | not met (88.6%) | met (4.2%) | not met (14.2%, CI high 15.7%) | met (0/100) | not evaluable |

The voltage reason applies to every row: the arms were not rerun on a

feeder under DC sessions. The 34-node feeder is out of band when idle, and

the node_123 result (closure) used Dutch durations.

Part 2 — Implementation

`ev2gym_thesis/demand/dc_sessions.py` (new)

The DC session-duration transform, applied from outside the library. No file

in ev2gym/ was edited.

The hook. EV2Gym has no session-duration key, and no hook between "the

population is generated" and "the episode runs" other than the spawner

itself. Rewriting departures after EV_spawner returns would keep the port

occupancy that decided which arrivals exist: an arrival blocked behind a

6-hour Dutch session would stay blocked behind a 42-minute one. The

transform therefore works inside the spawner's own loop:

1. It wraps loaders.EV_spawner, the name that load_ev_profiles calls.

2. Pass 1 runs the library spawner unchanged. This is the

Dutch-duration population every registry row used.

3. It restores the RNG state, then runs pass 2: the library spawner

again, with utils.spawn_single_EV temporarily replaced by a function

that:

for an arrival pass 1 also produced (same station, port and step),

returns pass 1's EV object itself, with only time_of_departure

rewritten. Arrival time, battery at arrival and desired capacity are

bitwise unchanged;

for an arrival that exists only because a port is now free, calls the

library's own spawn_single_EV under an RNG keyed on (scenario,

station, port, step). The energy requested is then the same in every

duration variant;

draws the connection duration from a lognormal with the target mean

and CV 0.5, using z ~ N(0,1) keyed the same way. The same z is used in

every variant, so the 32/42/78-minute populations are comonotone;

rounds the duration to the 15-minute step, half up, with a floor of 1

step, and applies the library's own horizon rule.

4. It sets the global RNG to its state after pass 1. Everything EV2Gym draws

after the spawner is therefore unchanged.

Setpoint guard. utils.generate_power_setpoints spreads each EV's

energy over [arrival + 1, departure), which is empty for a 1-step session,

so min() raises. For that computation only, a 1-step session is treated

as 2 steps and then restored. This affects the tracking target and EV2Gym's

Round Robin (which follows the setpoint) for 1-step sessions: 13% at 42 min,

29% at 32 min, 1% at 78 min. It is declared in chapter 08.

Parameters. They live in a sidecar <config>.dwell.json, never in a new

YAML key: EV2Gym does not consume one (CLAUDE.md config rule).

`ev2gym_thesis/demand/censoring.py` (extended)

The rejected-arrival replay now wraps whatever spawner is installed before

enable(). When the DC transform is on, the replay re-walks the transformed

population. Without it, behaviour is unchanged (the existing closure tests

still pin this).

`ev2gym_thesis/heuristics.py` (new)

RoundRobinTransformerCapped is a diagnostic arm. It keeps EV2Gym's Round

Robin allocation unchanged (same buffer, rotation and per-port action) and

replaces only the power budget: 0.999 × the transformer's rated power,

instead of the power setpoint. The setpoint itself is restored after each

call, so the tracking target is unchanged. It was added after the first C1

results showed that EV2Gym's Round Robin follows a median-smoothed setpoint

and never reads the transformer limit.

`scripts/dwell_session_stats.py` (new)

Part A and the realised-mean check of Part B. It builds the population

exactly as evaluation runs do (make_env + reset_for_evaluation, with the

censoring replay on), for 50 seeds × 2 days per session model. Outputs:

results/dwell_sessions_ev_level.csv;

results/dwell_sessions_by_cell.csv;

results/dwell_a_session_summary.csv;

results/dwell_a_reference_comparison.csv.

`scripts/run_dwell_capacity.py`, `scripts/run_dwell_worker.cmd` (new)

Configs. Each is the non-grid reference YAML with only three lines

changed: spawn_multiplier, number_of_charging_stations and

transformer.max_power. Each has its .dwell.json sidecar. They are

written once by --prepare:

experiments/phase3_infra_replicability/configs/dwell/, 81 configs;

run plans in .../dwell/plans/dwell_main.json and dwell_diag.json.

Row builders. The same real ones as before:

backfill_registry.run_single for the heuristics;

run_week7_grid.run_cell for the final RL model, with its notes

carrying out_of_training_distribution=dc_session_durations.

Workers and outputs.

Workers are detached and resumable, with 2 processes at most (the laptop

limit). Each writes a row shard and a censoring shard.

--merge writes results/dwell_registry.csv (the master registry

schema, 23,200 rows) and results/dwell_censoring_by_cell.csv.

`results/master_results.csv` is never opened for writing (a labelled

choice: the DC model is a different population model, and a separate file

removes any risk to existing rows and existing pins).

`scripts/analyze_dwell_capacity.py` (new)

Post-processing only. It computes:

per-run DNS (lower and upper bounds), satisfaction counting rejected

arrivals, peak power from the saved time series, and margin at Bogotá

prices;

the breaking criteria on the hardest CI side;

the C1 thresholds per arm and session model;

the C2 feasibility grid and its Pareto-minimal configurations;

the Part D physical units;

the E3 compliance table.

`scripts/export_dwell_results_xlsx.py` (new), `scripts/make_figures.py` (extended), `ev2gym_thesis/figures.py` (style appended)

Every dwell table has an .xlsx twin produced through

export_formatted_xlsx.

Figures f22–f24 were added (--only f22,f23,f24).

The transformer-aware Round Robin's style was appended without reassigning any existing

colour.

`scripts/extend_progress_log.py` (extended)

A new --section dwell appends section 16. Nothing already in the file is

modified.

Tests: `ev2gym_thesis/tests/test_dwell.py` (new)

Each test calls the production functions:

Arrivals and energy (B.2 test 1). The arrival draw matrix is

identical. Every shared arrival keeps its arrival time, battery at

arrival and desired capacity bitwise. Pass 1 equals the library

population. Energy is identical across the 32/42/78 variants.

Realised mean (B.2 test 2). It is within 5% of the target over the

100 cells, built by the production statistics script.

Transform disabled (B.2 test 3). AFAP, Round Robin and the final RL

model reproduce existing registry rows exactly.

Further checks. The setpoint guard, the config/sidecar contract, the

Part A pin, the dwell-registry schema and grid completeness, and that the

master registry carries no dwell rows.

References (APA 7)

Blu Radio. (2026, May 13). *Conductores en Bogotá podrán cargar hasta el 50 % de batería de su carro eléctrico en menos tiempo* (C. Durán, Author). https://www.bluradio.com/motor/conductores-en-boogta-podran-cargar-hasta-el-50-de-bateria-de-su-carro-electrico-en-menos-tiempo-so35

Enel Colombia. (2024, May). *Avances en infraestructura de recarga de vehículos eléctricos.* https://www.enel.com.co/es/historias/archive/2024/05/infraestructura-de-recarga-de-vehiculos-electricos.html

Hardman, S. (2026). Exploring electric vehicle driver activities and expenditure while using DC fast chargers. *Findings.* https://doi.org/10.32866/001c.162484

U.S. Department of Energy, Vehicle Technologies Office. (2023, December 4). *FOTW #1319: EV charging at paid DC fast charging stations average 42 minutes per session* [Fact of the Week]. https://www.energy.gov/cmei/vehicles/articles/fotw-1319-december-4-2023-ev-charging-paid-dc-fast-charging-stations-average